Inspection Checklist (based on Bronco6g OP repetitive feedback):

## Mechanical

- Pulley rub/ proximity next to radiator hose

- Coolant hose clamp tight

- Battery terminals tight (negative loose)

- Turbo Charge Air Cooler Hose securely attached to the Charge Air Cooler Pipe (these are the big black tubes in the engine compartment)

- Roll all windows up at once

- Check window auto-drop for excessive snag/wobble.  Reset windows by holding switches down for 10 secs.

- Tailgate hinge area and fuel door has paint issues

- Fluids all full (oil, washer, brake)

- Confirm SEVEN Quarts of oil filled, not six. It's a 7 qt pan with a 6 qt dipstick. Ref: https:///forum/threads/bronco-team-engineering-confirms-7-0-quarts-for-2-7l-engine-oil-change-is-correct.29047/

- Strut on the back window unglued from the hardtop

- Check all three bolts in the fuse box for looseness

- Alternator belt installed backwards (smooth side against pulley)

- Transfer case wire harness rubbing against drive shaft (missing plastic clamp)

## Paint/Exterior

- MIC Hardtop rough/sharp edges, gaps, leaks, roof rattles, popping from guide pins, latches...especially seal in middle

- MIC panels all match in color, not two-tone

- MIC Panels contacting, rubbing body panels, especially around rear corner panels/tailgate.  Source of squeaks.

- Paint bubbles/paint defects

- Paint overspray on windshield and body panels.  (Rough to touch vs. baby/glass smooth)

- Missing paint in door jambs/door brackets (may need to remove doors to see hinges in jambs), tailgate, fuel door

- Body panel alignment

- Hood dents on passenger side caused by excessive panel bond on cross bars

- No license plate mounting (if mounting alternate location)

- Check underneath for corrosion

- Check for discoloration of trim MIC top due to long stay in outdoor storage

- Check for transport damage

Interior

- Test all roof latches (some missing)

- Check seat stitching and ensure seats match

- Check seat belt webbing for defects

- Sound deadening installed (hardliner)

- Interior and trunk lights/bulbs work

- Owner’s Manual and Tool kit in glovebox

- Any TSBs/ Service Alerts? A-pillar leak TSB: 21-2403

- Whistling mirrors

- Ensure floor mats are as ordered (Raptor/Warthog/Ranger logo mats installed in Bronco)

## Miscellaneous

- Door bags

- Mismatched roll bars...holes drilled on one, but not the other

- Ensure all cameras work

- Check windshield washer bottle neck for zip-tie support (should not be there)

- Test Windshield Wiper reservoir is full and sprays fluid on windshield

Dealer Signature:_________________________________________ Date: _______________

Owner Signature: ________________________________________ Date: _______________